## Design and Implementation of a Wireless-Controlled       Robotic Car with Manual and PID Motion Control

Prepared by: Judy Sameh

# Abstract

This report presents the design and implementation of a wireless-controlled robotic car based on an ATmega8A microcontroller. The system allows a user to manually control the vehicle through directional commands, including forward, backward, left, right, and stop, while also providing speed adjustment. The control commands are transmitted from a graphical user interface through the communication system to an HC-05 Bluetooth module and finally to the ATmega8A microcontroller.

The microcontroller processes the received commands and controls two DC motors through a motor driver circuit. The direction of each motor is controlled using digital output signals, while the motor speed is controlled using Pulse Width Modulation (PWM). Independent control of the left and right motors allows the vehicle to perform both straight-line motion and turning maneuvers.

In addition to manual control, the project includes a proposed PID-based motion control system. An Inertial Measurement Unit (IMU) provides orientation feedback that can be used to compare the desired direction of the vehicle with its actual direction. The resulting error is processed by a Proportional-Integral-Derivative (PID) controller, which calculates a correction value. This correction is applied by adjusting the PWM values of the left and right motors, allowing the vehicle to compensate for directional drift.

The overall system integrates wireless communication, embedded programming, motor control, safety mechanisms, sensor feedback, and closed-loop PID control. The proposed design provides a foundation for developing a more stable and accurate autonomous or semi-autonomous robotic vehicle.

# 1. Introduction

Wireless robotic vehicles are widely used in educational, industrial, and research applications. These systems require an effective combination of communication, embedded control, motor actuation, and feedback mechanisms.

The purpose of this project is to develop a wireless-controlled car capable of receiving movement commands from a control station and converting these commands into physical motion. The vehicle uses an ATmega8A microcontroller as the main processing unit. The microcontroller receives commands through an HC-05 Bluetooth module and controls two DC motors through a motor driver circuit.

The basic control system provides manual movement in the forward, backward, left, and right directions, in addition to a stop function. The system also allows the speed of the vehicle to be adjusted through PWM signals.

A major challenge in a two-motor vehicle is maintaining a straight direction. Even when both motors receive the same PWM value, they may not rotate at exactly the same speed. This can cause the vehicle to drift away from its intended direction. Therefore, a PID-based feedback system using an IMU is proposed to improve directional stability.

The PID controller continuously compares the desired direction with the actual direction measured by the IMU. When an error occurs, the controller calculates a correction and adjusts the motor speeds accordingly.

# 2. Project Objectives

The main objective of this project is to design and implement a wireless-controlled robotic car with manual movement and the capability to support PID-based motion correction.

The specific objectives are:

- To establish wireless communication between the control station and the robotic car.

- To receive and process movement commands using an ATmega8A microcontroller.

- To control the direction of two DC motors.

- To control the speed of the motors using PWM.

- To provide forward, backward, left, right, and stop movements.

- To implement a communication timeout mechanism for safety.

- To integrate an IMU as a source of motion and orientation feedback.

- To implement or support a PID controller for automatic correction of directional errors.

- To independently adjust the speed of the left and right motors.

# 3. System Architecture

The proposed system architecture consists of several software and hardware components.

The overall control flow is:

React GUI
↓
WebSocket
↓
Python / ROS Backend
↓
Serial Bluetooth Communication
↓
HC-05 Bluetooth Module
↓
UART Communication
↓
ATmega8A Microcontroller
↓
Motor Control Logic
↓
Motor Driver Circuit
↓
Left and Right DC Motors

The operator interacts with the graphical user interface and selects the desired movement and speed. The selected command is transferred through the communication system to the Bluetooth module.

The HC-05 module sends the command serially to the ATmega8A microcontroller. The microcontroller processes the received command and generates the necessary digital and PWM signals. These signals are transmitted to the motor driver circuit, which controls the electrical power supplied to the motors.

The motor driver performs the high-current switching operation required by the DC motors, while the ATmega8A provides the low-power control signals.

# 4. Hardware Design

## 4.1 ATmega8A Microcontroller

The ATmega8A acts as the main controller of the vehicle.

Its main functions include:

- Receiving wireless commands.

- Processing movement instructions.

- Controlling motor direction.

- Generating PWM signals for speed control.

- Monitoring communication activity.

- Stopping the motors when communication is lost.

- Supporting future integration with the IMU and PID controller.

The selected pin assignments are shown in Table 1.

### Table 1. ATmega8A Pin Assignment

| ATmega8A Pin | Assigned Function |
| --- | --- |
| PD0 / RXD | Receive data from HC-05 |
| PD1 / TXD | Transmit data to HC-05 |
| PB0 | Left Motor Direction Input 1 |
| PB1 | Left Motor Direction Input 2 |
| PB2 | Right Motor Direction Input 1 |
| PB3 | Right Motor Direction Input 2 |
| PD5 / OC1A | Left Motor PWM |
| PD4 / OC1B | Right Motor PWM |

The use of PD0 and PD1 allows the system to use the hardware UART communication interface of the ATmega8A.

The pins PD4 and PD5 are used for PWM-based speed control, allowing independent adjustment of the left and right motor speeds.

## 4.2 HC-05 Bluetooth Module

The HC-05 Bluetooth module provides the wireless communication link between the control system and the ATmega8A microcontroller.

The UART connection is arranged as follows:

HC-05 TXD → ATmega8A PD0/RXD

HC-05 RXD ← ATmega8A PD1/TXD

The transmitter and receiver pins are crossed because the transmitting output of one device must be connected to the receiving input of the other device.

The serial communication is initialized at a baud rate of 9600 bits per second.

## 4.3 Motor Driver Circuit

The motor driver circuit acts as the interface between the microcontroller and the DC motors.

The ATmega8A cannot directly supply the current required by the motors. Therefore, it sends control signals to the motor driver, while the motor driver handles the electrical power required for motor operation.

The control relationship can be represented as:

ATmega8A
↓
Digital and PWM Control Signals
↓
Motor Driver / H-Bridge
↓
Motor Current and Direction
↓
DC Motors

The motor driver controls both the direction and speed of each motor.

# 5. Motor Direction Control

Each DC motor requires two direction-control signals.

For the left motor, the direction is controlled using PB0 and PB1.

### Table 2. Left Motor Direction Control

| PB0 | PB1 | Motor Action |
| --- | --- | --- |
| HIGH | LOW | Forward |
| LOW | HIGH | Backward |
| LOW | LOW | Stop |

The right motor uses the same control principle through PB2 and PB3.

### Table 3. Right Motor Direction Control

| PB2 | PB3 | Motor Action |
| --- | --- | --- |
| HIGH | LOW | Forward |
| LOW | HIGH | Backward |
| LOW | LOW | Stop |

The two direction signals control the switching action of the H-Bridge. Changing the active signal changes the direction of electrical current through the motor. Reversing the current causes the motor to rotate in the opposite direction.

The motor control chain can therefore be described as:

ATmega8A Direction Pins
↓
H-Bridge Control Signals
↓
Current Direction Changes
↓
Motor Rotation Direction Changes

# 6. Motor Speed Control Using PWM

The speed of the motors is controlled using Pulse Width Modulation.

The selected PWM outputs are:

- PD5 for the left motor.

- PD4 for the right motor.

PWM allows the microcontroller to control the effective electrical power applied to each motor.

The implemented speed levels are converted into PWM values as shown in Table 4.

### Table 4. Speed Conversion

| Speed Input | PWM Value | Speed Level |
| --- | --- | --- |
| 3 | 80 | Low |
| 6 | 160 | Medium |
| 9 | 255 | High |

Independent PWM control is important because the left and right motors can later be adjusted separately for steering and PID-based correction.

# 7. Manual Control System

## 7.1 Command Protocol

The vehicle receives movement commands through the Bluetooth communication system.

The command format is:

M,Direction,Speed

For example:

M,F,6

In this command:

- M represents a movement command.

- F represents forward movement.

- 6 represents the selected speed.

The supported movement directions are shown in Table 5.

### Table 5. Movement Commands

| Command | Function |
| --- | --- |
| F | Forward |
| B | Backward |
| L | Left |
| R | Right |
| S | Stop |

## 7.2 Command Processing

The firmware receives serial characters and stores them in a command buffer.

When a complete command is received, the program performs the following operations:

- Terminates the received command string.

- Checks whether the command is valid.

- Sends the command to the command-processing function.

- Extracts the movement direction.

- Extracts the selected speed.

- Executes the appropriate movement function.

The command-processing structure separates communication from motor control, making the program easier to organize and maintain.

# 8. Software Design

The firmware is organized into several main sections:

- Pin definitions and variable declarations.

- System initialization.

- Continuous command reception.

- Command handling.

- Movement execution.

- Motor control functions.

- Speed conversion.

- Communication safety monitoring.

The major functions are:

- handleCommand()

- executeMove()

- moveForward()

- moveBackward()

- moveLeft()

- moveRight()

- stopMotors()

- convertSpeed()

This modular design improves readability and allows each function to perform a specific task.

# 9. Movement Functions

## 9.1 Forward Motion

During forward motion, both motors rotate in the forward direction.

The sequence is:

Receive Forward Command
↓
Convert Speed Input to PWM
↓
Set Left Motor to Forward
↓
Set Right Motor to Forward
↓
Apply PWM to Both Motors

Both motors receive the selected speed value.

## 9.2 Backward Motion

For backward movement, the direction signals of both motors are reversed.

The motors then rotate in the opposite direction while receiving the selected PWM value.

## 9.3 Left Turn

For the implemented left-turn operation:

- The left motor is stopped.

- The right motor moves forward.

This difference in wheel movement causes the vehicle to turn toward the left.

## 9.4 Right Turn

For the implemented right-turn operation:

- The right motor is stopped.

- The left motor moves forward.

This causes the vehicle to turn toward the right.

## 9.5 Stop Function

The stop function disables the movement of both motors.

The direction pins are set to their stop condition, and the PWM values are set to zero.

The stop function is activated when:

- The operator sends a stop command.

- The communication timeout is reached.

# 10. Communication Safety Mechanism

The system includes a communication safety feature to prevent uncontrolled vehicle motion.

A timer is associated with the reception of valid commands.

After each received command, the system updates the last command time.

The program continuously checks whether:

Current Time − Last Command Time > Communication Timeout

If no command is received within the specified timeout period of 1000 ms, the program assumes that the communication link has been lost.

The system then executes:

stopMotors()

The safety sequence is:

Communication Lost
↓
No New Command Received
↓
Timeout Reached
↓
Motors Stopped

This mechanism prevents the car from continuing to move when the communication link is interrupted.

# 11. PID Motion Control

## 11.1 Motivation for PID Control

During manual forward movement, the two motors may receive the same PWM value.

For example:

Left Motor PWM = 160

Right Motor PWM = 160

However, equal PWM values do not necessarily produce identical motor speeds.

The vehicle may drift because of:

- Differences between the motors.

- Different friction conditions.

- Battery voltage changes.

- Uneven surfaces.

Therefore, a feedback system is required to detect directional errors and correct them automatically.

The proposed solution is a PID controller using feedback from an IMU.

## 11.2 IMU Feedback

IMU stands for Inertial Measurement Unit.

The IMU can contain:

- An accelerometer.

- A gyroscope.

- A magnetometer, depending on the sensor.

The accelerometer measures acceleration.

The gyroscope measures rotational motion.

The magnetometer can assist with heading determination.

The primary orientation values are:

- Roll: tilt from left to right.

- Pitch: tilt from forward to backward.

- Yaw: rotation toward the left or right.

For directional correction of the vehicle, the heading or yaw information is particularly important.

# 12. PID Control Methodology

The PID controller operates using a closed-loop feedback system.

The general process is:

Desired Direction
↓
Compare with Actual Direction from IMU
↓
Calculate Directional Error
↓
PID Controller
↓
Calculate Correction
↓
Adjust Left and Right Motor PWM
↓
Correct Vehicle Direction

The system continuously repeats this process while the vehicle is moving.

## 12.1 Target and Actual Direction

When the car receives a forward command, the system can store the current direction as the target direction.

The IMU then continuously measures the actual orientation of the vehicle.

For example:

Target Direction = 40°

Actual Direction = 45°

The controller compares these two values.

## 12.2 Error Calculation

The directional error is calculated as:

Error = Target Direction − Actual Direction

Using the previous example:

Error = 40° − 45°

Therefore:

Error = −5°

The error represents the difference between the direction the vehicle should follow and the direction it is actually following.

This error is supplied to the PID controller.

## 12.3 PID Controller

The PID controller consists of three components:

### Proportional Component

The proportional component responds to the current value of the error.

### Integral Component

The integral component considers the accumulated error over time.

### Derivative Component

The derivative component considers how quickly the error changes.

The overall controller output is:

PID Output = P + I + D

The PID output represents the required correction to the motor control system.

# 13. Motor Correction Using PID

The PID correction is applied by changing the PWM values of the left and right motors.

For example:

Base PWM = 160

PID Correction = 20

The corrected motor outputs may become:

Left PWM = 160 + 20 = 180

Right PWM = 160 − 20 = 140

The two motors now operate at different speeds.

The resulting sequence is:

PID Correction
↓
Different PWM Values
↓
Different Motor Speeds
↓
Vehicle Rotates
↓
Vehicle Moves Toward the Target Direction

The exact motor that must be increased or decreased depends on the direction of the error and the physical motor wiring.

# 14. Proposed PID Software Algorithm

The PID software implementation requires the following major stages:

- Initialize the IMU communication.

- Define the PID constants.

- Define the PID variables.

- Initialize the motor-control system.

- Read the IMU continuously.

- Determine the current vehicle orientation.

- Store or determine the target direction.

- Calculate the directional error.

- Calculate the elapsed time.

- Calculate the proportional term.

- Calculate the integral term.

- Calculate the derivative term.

- Calculate the total PID correction.

- Modify the left and right PWM values.

- Apply the corrected PWM values to the motors.

- Repeat the process continuously.

The main control relationship can be represented as:

IMU Measurement
↓
Actual Direction
↓
Compare with Target Direction
↓
Calculate Error
↓
PID Calculation
↓
PWM Correction
↓
Motor Speed Adjustment
↓
Vehicle Direction Correction

# 15. Overall System Operation

The complete operation of the wireless-controlled vehicle can be summarized as follows:

- The user selects a movement command from the GUI.

- The command is transmitted through the software communication system.

- The command reaches the HC-05 Bluetooth module.

- The HC-05 sends the command to the ATmega8A through UART.

- The ATmega8A receives and processes the command.

- The movement direction and speed are extracted.

- The appropriate motor-control function is selected.

- Digital signals determine motor direction.

- PWM signals determine motor speed.

- The motor driver supplies the required electrical power.

- The left and right motors produce vehicle movement.

- During PID operation, the IMU measures vehicle orientation.

- The system compares the actual direction with the target direction.

- A PID correction is calculated.

- The left and right motor PWM values are adjusted.

- The process repeats continuously to maintain the desired direction.

# 16. Conclusion

This report presented the design of a wireless-controlled robotic car based on an ATmega8A microcontroller.

The manual control system allows the operator to control the vehicle through forward, backward, left, right, and stop commands. The system also supports speed adjustment through PWM control.

Wireless communication is achieved using an HC-05 Bluetooth module connected to the ATmega8A through UART communication. The microcontroller processes the received commands and generates the required direction and speed signals.

The motor driver acts as an interface between the low-power control signals produced by the microcontroller and the electrical power required by the DC motors. The use of two independently controlled motors allows the vehicle to perform both linear and turning movements.

A communication timeout mechanism is also included to improve safety. If communication is lost for more than the specified period, the system automatically stops the motors.

The proposed PID motion control system extends the capabilities of the vehicle by adding sensor-based feedback. An IMU can measure the actual orientation of the vehicle, allowing the system to calculate the difference between the target and actual direction. The PID controller then generates a correction that can independently modify the left and right motor PWM values.

The overall system provides a foundation for a more advanced robotic vehicle capable of combining manual control with feedback-based motion stabilization.

# 17. Recommendations for Future Development

The following improvements can be considered for future development of the project:

- Integrate the IMU directly into the complete firmware.

- Experimentally tune the PID constants.

- Add sensor calibration before vehicle operation.

- Add wheel encoders for improved speed feedback.

- Improve the turning algorithm using proportional differential motor speeds.

- Add battery voltage and current monitoring.

- Integrate sensor feedback with the ROS communication system.

- Add autonomous navigation functions.

- Add obstacle detection.

- Develop a real-time dashboard for monitoring the vehicle status.